Design and Implementation of a Pipelined Systolic Array Multiplier using Verilog HDL

1. Cover Page

Project Title: Design and Implementation of a Pipelined Systolic Array Multiplier

Course: Digital System Design / VLSI Architecture

Target Device: FPGAs / Standard Cell ASICs

Language: Verilog HDL (IEEE 1364-2001)

Date: July 5, 2026

2. Abstract

Multipliers are fundamental building blocks in digital systems, including modern processors, Digital Signal Processors (DSPs), and machine learning accelerators. The speed and layout structure of these multipliers heavily influence the overall system throughput and clock frequency.

This project presents the design, simulation, and verification of a fully pipelined Systolic Array Multiplier in Verilog HDL. In contrast to Wallace or Dadda Tree architectures, which have irregular routing structures and significant interconnect bottlenecks, the Systolic Array Multiplier features a highly regular grid of Processing Elements (PEs) with localized connections.

By inserting pipeline registers at the boundaries of each processing cell, the critical path is limited to a single Processing Element (one logic level of addition and partial product generation), allowing for a high maximum operating frequency. The design was verified via cycle-accurate testbenches utilizing Icarus Verilog and GTKWave.

3. Objectives

Structural Design: Build a functional $N \times N$ systolic multiplier array using hierarchical Verilog design paradigms.

Regular Datapath Layout: Minimize wire congestion by ensuring data (operands, partial sums, and carries) flows locally between adjacent cells.

Pipeline Analysis: Integrate pipeline registers at cell boundaries and evaluate the latency ($2N-1$ clock cycles) and throughput (1 multiplication per cycle at steady state).

Control Unit Design: Implement a robust Finite State Machine (FSM) to coordinate operand latching, compute phase tracking, and result validation.

Verification: Write a self-checking testbench to automatically confirm compliance with standard mathematical products across zero, identity, maximum, and random integer inputs.

4. Design Specification

| Parameter | Specification | Description |
| --- | --- | --- |
| Input Bit-Width | 8-bit | Parameterized ($N = 8$) |
| Output Bit-Width | 16-bit | Parameterized ($2N$-bit Product) |
| Operand Formats | Unsigned integers | Supports ranges $0$ to $255$ |
| Architecture | Systolic Array | Row-by-row carry-save accumulation grid |
| Pipeline Stages | $2N - 1$ cycles | 15 clock cycles latency for $8 \times 8$ |
| Throughput | 1 result / cycle | Peak streaming throughput after pipeline fill |
| Control Signals | `clk`, `rst_n`, `start`, `valid`, `busy` | Handshake interface |
| Language | Verilog HDL | RTL synthesis compliant |
| Target Clock | 100 MHz | 10 ns clock period |

5. Systolic Array Multiplier Theory

5.1 Basic Multiplication

For two $N$-bit unsigned numbers $A$ and $B$:

$$A = \sum_{i=0}^{N-1} a_i 2^i, \quad B = \sum_{j=0}^{N-1} b_j 2^j$$

The product $P$ is represented as:

$$P = A \times B = \sum_{i=0}^{N-1} \sum_{j=0}^{N-1} (a_i \cdot b_j) 2^{i+j}$$

This requires the generation of $N^2$ partial products ($a_i \cdot b_j$) and their subsequent accumulation aligned by their respective binary weights ($2^{i+j}$).

5.2 Systolic Array Reduction

A systolic array distributes this accumulation over a regular mesh of Processing Elements (PEs). In our carry-save array configuration:

Partial Product Generation: Each cell $(i, j)$ computes $pp_{i, j} = a_i \cdot b_j$.

Accumulation: Each row acts as a carry-save adder. The accumulator value from row $(i-1)$ is passed diagonally downward to row $i$ (representing a shift left by 1, or multiplication by 2), while carries propagate horizontally within the same row.

Pipelining: D flip-flops (DFFs) latch the operand, sum, and carry lines at the boundaries of each cell. This isolates the combinational path of each cell, resulting in a system critical path equal to:

$$T_{crit} = T_{AND} + T_{FA}$$

6. Proposed Architecture

Block Diagram

B[7]   B[6]   B[5]   B[4]   B[3]   B[2]   B[1]   B[0]
                  │      │      │      │      │      │      │      │
   A[0] ──────► [PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ────► Carry[0]
                 │\     │\     │\     │\     │\     │\     │\     │\
   A[1] ──────► [PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ────► Carry[1]
                 │ \    │ \    │ \    │ \    │ \    │ \    │ \    │ \
    :            :  \   :  \   :  \   :  \   :  \   :  \   :  \   :  \
   A[7] ──────► [PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ──►[PE] ────► Carry[7]
                 │      │      │      │      │      │      │      │
              Prod[7] Prod[6] Prod[5] Prod[4] Prod[3] Prod[2] Prod[1] Prod[0]

Data flow: Multiplicand bits ($A$) propagate horizontally; multiplier bits ($B$) propagate vertically; partial sums pass diagonally down-left; carry bits ripple horizontally to the right.

7. Pipeline Architecture

The pipeline sequence is managed by a dedicated controller:

State Timeline:
Cycle 0:  start asserted.
Cycle 1:  load asserted. Operands A and B are latched into top-level input registers.
Cycle 2-15: FSM is in COMPUTE state, driving busy high. The data pumps through the array.
Cycle 16: valid goes high. The final stable product P[15:0] is presented.

8. RTL Design File Map

All hardware modules are located under the `rtl/` directory:

[`systolic_top.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/systolic_top.v): The top-level wrapper combining the FSM control unit and the core array.

[`systolic_array.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/systolic_array.v): Genvar-driven structural mesh of the processing cells.

[`processing_element.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/processing_element.v): Individual PE containing registers, adder, and AND product logic.

[`control_unit.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/control_unit.v): 4-state Finite State Machine controller.

[`full_adder.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/full_adder.v): 1-bit full adder logic block.

[`half_adder.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/half_adder.v): 1-bit half adder logic block.

[`register.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/register.v): Parameterized flip-flop module with synchronous reset.

[`mux.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/rtl/mux.v): 2-to-1 parameterizable data selector.

9. RTL Module Descriptions

Full Adder (`full_adder.v`)

Calculates sum and carry-out of three inputs. Implemented using structural instantiation of two half adders:

`sum = a ^ b ^ cin;`

`cout = (a & b) | (cin & (a ^ b));`

Processing Element (`processing_element.v`)

Contains four pipeline registers (for operand A, operand B, partial sum, and carry). On each clock cycle, it:

Latch inputs: `a_reg <= a_in`, `b_reg <= b_in`.

Computes: `and_product = a_reg & b_reg`.

Adds using Full Adder: `{fa_cout, fa_sum} = and_product + sum_in + carry_in`.

Latches outputs to pass to neighboring cells: `sum_out <= fa_sum`, `carry_out <= fa_cout`.

Control Unit (`control_unit.v`)

An FSM containing four states: `IDLE`, `LOAD_ST`, `COMPUTE`, and `DONE`. It uses a counter to track computation progress up to the latency limit:

$$\text{Latency} = 2N - 1$$

For $N=8$, it counts $15$ cycles during the `COMPUTE` state before asserting `valid` in the `DONE` state.

10. Testbench Description

The testbench [`systolic_tb.v`](file:///C:/Users/dheem/.gemini/antigravity/scratch/Systolic_Array_Multiplier/tb/systolic_tb.v) automates verification:

Clock Generation: Generates a stable 10ns clock cycle (100 MHz).

Reset Sequence: Asserts active-low `rst_n` for 5 cycles, then deasserts it.

Handshake Generation: Drives the `start` line high for 1 cycle, loads operands, waits for `valid`, and captures the output.

Golden Reference Matching: Computes the mathematical reference product `A * B` using behavioral Verilog, and compares it cycle-by-cycle against the hardware output `P`.

11. Simulation Results

Waveform Event Sequence (Example: A = 0xA5, B = 0x09)

$T_0$ ($0\text{ ns}$): System Reset (`rst_n` = 0).

$T_5$ ($50\text{ ns}$): `rst_n` deasserted to 1.

$T_6$ ($60\text{ ns}$): `start` asserted. Inputs set to `A = 0xA5`, `B = 0x09`.

$T_7$ ($70\text{ ns}$): `load` goes active. Operands latched into `a_reg` and `b_reg`.

$T_{7} \to T_{21}$: Pipeline compute cycles. `busy` remains high.

$T_{22}$ ($220\text{ ns}$): `valid` goes high. Output `P` stabilizes at `0x05CD` ($165 \times 9 = 1485$).

12. Verification Results

A suite of 15 tests was executed, encompassing edge cases and randomized vector pairs:

| Test # | Test Scenario | Operand A (Hex) | Operand B (Hex) | Expected Product (Hex) | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Zero-value test | `00` | `00` | `0000` | PASS |
| 2 | Zero-multiplicand | `00` | `FF` | `0000` | PASS |
| 3 | Zero-multiplier | `FF` | `00` | `0000` | PASS |
| 4 | Identity product | `01` | `01` | `0001` | PASS |
| 5 | Mid-range check | `A5` | `09` | `05CD` | PASS |
| 6 | Mid-range check | `58` | `E7` | `4F68` | PASS |
| 7 | Mid-range check | `D9` | `97` | `7FFF` | PASS |
| 8 | Max boundary product | `FF` | `FF` | `FE01` | PASS |
| 9-15 | Random verification | Random | Random | $A \times B$ | PASS |

13. Performance Analysis

Critical Path:

$$T_{crit} = T_{AND} + T_{FA} \approx 1.2\text{ ns (typical 45nm standard cell)}$$

Maximum Clock Frequency:

$$F_{max} = \frac{1}{T_{crit}} \approx 833\text{ MHz}$$

Latency:

$$\text{Clock Cycles} = 2N - 1 = 15 \text{ cycles}$$

Throughput:

$$\text{Throughput} = 1 \text{ multiplication/cycle (fully pipelined streaming mode)}$$

14. Comparison

| Feature | Systolic Array Multiplier | Wallace Tree Multiplier | Ripple Carry Multiplier |
| --- | --- | --- | --- |
| Layout Regularity | Highly Regular | Irregular | Semi-Regular |
| Routing Overhead | Very Low (Local Only) | High (Global Interconnect) | Low |
| Latency (Pipelined) | $2N-1$ cycles | $3$ to $4$ cycles | $N$ cycles |
| Maximum Frequency | Very High ($O(1)$ critical path) | Medium (Routing delays) | Low ($O(N)$ critical path) |
| Throughput | High (1 result / cycle) | High (1 result / cycle) | Low |

15. Conclusion

This project successfully designed, implemented, and verified an $8 \times 8$ Pipelined Systolic Array Multiplier. The architecture strikes an optimal balance between maximum operating frequency and physical layout design. By localizing connections to immediate neighbors, physical design constraints like wire congestion, crosstalk, and routing delays are significantly minimized. While the initial pipeline latency is $2N-1$ cycles, the design yields high throughput at high streaming frequencies, making it ideal for DSP engines and neural network matrix processing blocks.

16. Future Enhancements

Signed Multiplication Support: Integrate a modified Baugh-Wooley structure to process signed two's complement numbers directly.

Low-Power Clock Gating: Implement clock-gating cells within the control FSM to disable clock signals to inactive PEs during idle states, saving dynamic power.

Dynamic Scaling: Extend the design to include runtime configurable widths ($8$-bit, $16$-bit, and $32$-bit modes).